WatchGuard Technologies

Network Security

Firebox T Series

Tabletop Network Security Appliances for Small and Mid-Size Offices

User Manual — Compiled from Official WatchGuard Documentation

July 2026

# 1. Product Overview

The WatchGuard Firebox T Series is a family of compact, tabletop Unified Threat Management (UTM) firewall appliances designed for small offices, home offices, branch locations, and distributed mid-market networks. The T Series is built and supported by WatchGuard for managed service providers (MSPs) and IT teams, and it delivers fast deployment, strong performance, and AI-driven threat detection in a small physical footprint.

The current tabletop line-up includes the T115-W, T125, T145, and T185 models, alongside earlier tabletop models still sold and supported, such as the T45-CW, NV5, and Wi-Fi variants (T115-W, T125-W, T145-W). Models are differentiated primarily by throughput, port count, memory, and wireless capability, allowing organizations to choose the right size of appliance for their network.

Every Firebox T Series appliance can be managed locally through Fireware Web UI/WatchGuard System Manager, or centrally through WatchGuard Cloud, which provides a single hub for policy configuration, monitoring, and reporting across many devices.

# 2. Key Features

- AI-powered threat detection integrated with WatchGuard ThreatSync (XDR), combining Firebox telemetry with endpoint and identity data.

- Unified Threat Management: firewall, VPN (Branch Office VPN, IPSec, SSL), gateway antivirus, intrusion prevention, and application control in one appliance.

- Integrated SD-WAN for optimizing and load-balancing multiple WAN connections and reducing reliance on costly MPLS or 4G/LTE links.

- High-speed Gigabit Ethernet ports; select models add Wi-Fi 6 or Wi-Fi 7 wireless and PoE for access points and IoT devices.

- Zero-touch, cloud-managed deployment through WatchGuard Cloud, plus locally-managed options through Fireware Web UI and WatchGuard System Manager (WSM).

- Built-in logging and reporting with 100+ dashboards and reports, including PCI and HIPAA compliance report templates.

- Transparent, bundled licensing (Basic Security Suite / Total Security Suite) with no hidden feature-based licensing traps.

- Energy-efficient, fanless or ultra-quiet metal chassis designs across the current generation.

- Optional WatchGuard NDR add-on that analyzes NetFlow data with machine learning to detect lateral movement, command-and-control traffic, and other hidden internal threats.

# 3. Hardware and Software Components

## 3.1 Hardware

| Component | Description |
| --- | --- |
| Firebox appliance | Tabletop chassis (e.g., T115-W, T125, T145, T185) with multiple Gigabit Ethernet ports. |
| Power adapter and cord | Included with the appliance; region-specific plug. |
| Wireless module (select models) | Built-in Wi-Fi radio on "-W" model variants (e.g., T115-W). |
| PoE ports (select models) | Available on models from the T35 class upward, used to power access points or IoT devices. |
| Mounting/desk feet | For tabletop placement; some models support optional rack-mount kits. |

## 3.2 Software

- Fireware OS — the operating system that runs on the Firebox and enforces firewall policy.

- Fireware Web UI — browser-based interface for local device configuration.

- WatchGuard System Manager (WSM) — Windows application for local management, including Policy Manager and the Quick Setup Wizard.

- WatchGuard Cloud — cloud-based management console for cloud-managed devices, licensing, and reporting.

# 4. System Requirements

| Requirement | Details |
| --- | --- |
| Network | Active internet connection (WAN/external interface) and a local network (trusted interface) with an available Ethernet or Wi-Fi connection. |
| Administration computer | A computer with an Ethernet network interface (or Wi-Fi, where supported) and a supported web browser. |
| Local management software | WatchGuard System Manager (Windows) if using the Quick Setup Wizard/Policy Manager. |
| Firmware | Fireware OS v12.5.7 or higher is required to connect a Firebox to WatchGuard Cloud as a cloud-managed device (v12.7.1+ for FireboxV). |
| WatchGuard account | A WatchGuard account is required to activate the appliance and, if used, a WatchGuard Cloud account for cloud management. |
| License/feature key | A valid Basic Security Suite or Total Security Suite subscription to enable full security services. |

Note: Requirements vary slightly by model and Fireware version. Always confirm current minimum Fireware and browser requirements in the WatchGuard Help Center before deployment.

# 5. Installation

## 5.1 Before You Begin

- Decide on a physical location for the appliance, near power and network cabling.

- Record your existing network IP address scheme (external, trusted, and any optional interfaces).

- Activate the Firebox: go to https://myproducts.watchguard.com/activate, log in to your WatchGuard account, and enter the appliance serial number to generate a feature key.

## 5.2 Physical Setup

- Connect Firebox interface 0 (external) to a network or modem with internet access.

- Power on the Firebox with factory-default settings.

- Connect Firebox interface 1 to your administration computer (or connect over Wi-Fi where supported), using a computer configured for DHCP.

## 5.3 Choosing a Management Method

- Cloud-Managed (recommended) — the Firebox is added to WatchGuard Cloud first; it then downloads its configuration automatically once connected.

- Locally-Managed — a new configuration is created directly on the device using the Web Setup Wizard or the WSM Quick Setup Wizard.

# 6. Initial Setup

## 6.1 Cloud-Managed Quick Start

- 1. In WatchGuard Cloud, add a new Firebox as a cloud-managed device and complete the Add Device wizard (device name, time zone, and network settings).

- 2. Start the Firebox with factory-default settings and connect interface 0 to a network with internet access.

- 3. The Firebox automatically connects to WatchGuard Cloud and downloads the configuration created in the wizard.

- 4. If the device cannot obtain an address through DHCP, open https://10.0.1.1:8080, log in with the default user name admin and passphrase readwrite, and use the Web Setup Wizard to select Cloud-Managed and enter external network settings manually.

- 5. Confirm the deployment status changes from "Waiting for Initial Connection" to a successful, connected state on the Deployment History page.

## 6.2 Locally-Managed Quick Start

- Open a browser and go to https://10.0.1.1:8080 while connected to interface 1.

- Log in with user name admin and passphrase readwrite (the Web Setup Wizard starts automatically on a factory-default device).

- Select Locally-Managed (Create New Configuration), then follow the wizard to set external and trusted interface settings, device passphrases, and a time zone.

Note: Change the default admin (readwrite) and status (readonly) passphrases immediately after setup, and use unique, strong passphrases for every Firebox.

# 7. Basic Configuration

- Network interfaces: configure mixed routing mode (each interface on its own subnet, using NAT) — the default and most common mode — or drop-in/bridge mode for specific network layouts.

- Firewall policies: the setup wizard automatically enables default, secure firewall policies; review and adjust policies for the applications and services your organization uses.

- Security subscriptions: enable licensed services such as Gateway AntiVirus, IntelligentAV, APT Blocker, Intrusion Prevention Service, spamBlocker, WebBlocker, and Application Control according to your Security Suite subscription.

- VPN configuration: set up Branch Office VPN (BOVPN) for site-to-site connectivity, and Mobile VPN (IPSec or SSL) for remote users.

- SD-WAN: configure multiple WAN interfaces or actions to load-balance or fail over traffic across available internet connections.

- Administrative accounts: create additional operator accounts with role-based permissions in WatchGuard Cloud for delegated administration.

# 8. Common Use Cases

| Use Case | How the Firebox T Series Helps |
| --- | --- |
| Small office / home office (SOHO) security | Entry-level models (e.g., T115-W) provide enterprise-grade UTM protection in a compact, Wi-Fi-capable unit. |
| Branch office and multi-site networks | Centralized management in WatchGuard Cloud and BOVPN connect distributed offices securely. |
| Remote and hybrid workforce access | Mobile VPN and, in combination with FireCloud, zero trust-based remote access. |
| Retail and point-of-sale environments | PCI-focused reporting templates and HTTPS content inspection support compliance needs. |
| MSP-delivered managed security | Bundled licensing, zero-touch deployment, and WatchGuard Cloud multi-tenant management simplify service delivery. |

# 9. Daily Operations

- Monitor device and network health from the WatchGuard Cloud Dashboard or locally through Fireware Web UI / Firebox System Manager.

- Review daily and historical traffic, threat, and VPN reports (100+ built-in dashboards and reports).

- Check Deployment History after any configuration change to confirm it was applied successfully.

- Review and respond to security event notifications generated by subscription services (e.g., IPS, Gateway AntiVirus, APT Blocker).

- Periodically verify that Fireware OS and signature updates are current.

- Rotate and manage administrator passphrases and operator roles as part of routine security hygiene.

# 10. Troubleshooting

| Issue | Recommended Action |
| --- | --- |
| Firebox will not connect to WatchGuard Cloud | Confirm the device is on factory-default settings, verify Fireware OS meets the minimum version (v12.5.7+), and check the Deployment History page for status. |
| Cannot reach the Web Setup Wizard at 10.0.1.1:8080 | Confirm the administration computer is configured for DHCP and is connected to the correct interface (interface 1, or the applicable management interface on larger models). |
| Configuration changes are not applied | Check the Deployment History page in WatchGuard Cloud; deployment may be pending or held until manually deployed. |
| No internet access after setup | Verify external interface network settings (Static IP, DHCP, or PPPoE) and cabling to interface 0. |
| Forgotten admin passphrase | The Firebox must be reset to factory-default settings and reconfigured using a setup wizard, or restored from a backup image. |

# 11. Best Practices

- Always change the default admin and status passphrases before placing the Firebox into production.

- Keep Fireware OS up to date to maintain compatibility with WatchGuard Cloud and to receive the latest security fixes.

- Use WatchGuard Cloud centralized management for multi-site or MSP-managed deployments to maintain consistent policy templates.

- Record your network IP address plan before and after configuration for easier troubleshooting.

- Enable the full set of licensed security services (IPS, Gateway AntiVirus, APT Blocker, WebBlocker) rather than firewalling alone.

- Regularly review compliance and traffic reports to detect abnormal usage patterns.

- Back up your configuration before making major changes, and store backup images securely.

# 12. Frequently Asked Questions

## Q: What is the difference between cloud-managed and locally-managed Fireboxes?

Cloud-managed Fireboxes are configured, monitored, and managed entirely from WatchGuard Cloud. Locally-managed Fireboxes are configured with Fireware Web UI or WatchGuard System Manager, and can optionally be added to WatchGuard Cloud for monitoring and reporting only.

## Q: Can I migrate a locally-managed Firebox to cloud management?

Yes. WatchGuard provides a documented migration path, though a new configuration is created in WatchGuard Cloud rather than a direct import of the existing local configuration.

## Q: Do all T Series models support Wi-Fi?

No. Wireless is available on "-W" model variants (for example, T115-W). Some higher-throughput models, such as the T185, are wired-only.

## Q: What is included in the Firebox license?

Firebox appliances are sold with a Basic Security Suite or Total Security Suite subscription, which bundles core services such as Gateway AntiVirus and spamBlocker, along with standard or premium support (LiveSecurity).

## Q: Where can I get official Firebox documentation?

The WatchGuard Help Center (watchguard.com/help/docs) hosts current Fireware and WatchGuard Cloud documentation, including setup guides, datasheets, and release notes.